Advanced Predictive Analytics and Multi-Criteria Decision Frameworks for Electric Vehicle Battery Life Cycle Management within Circular Supply Chains

The global shift toward vehicle electrification has fundamentally altered the industrial landscape, placing lithium-ion batteries at the epicenter of sustainable transport strategies. As the primary energy storage units for electric vehicles (EVs), these batteries are prized for their high energy density, power-to-weight ratios, and extensive cycle life. However, the proliferation of lithium-ion batteries introduces a critical challenge concerning their aging, degradation, and ultimate disposal. The transition from a linear "take-make-dispose" model to a circular supply chain is no longer a peripheral environmental goal but a central industrial imperative, driven by both the scarcity of critical raw materials and stringent regulatory mandates such as the European Union’s 2023/1542 Regulation.1 Within this context, the integration of advanced predictive modeling, specifically Long Short-Term Memory (LSTM) networks, and multi-criteria decision-making (MCDM) tools like the Technique for Order Preference by Similarity to Ideal Solution (TOPSIS), serves as a robust framework for managing the transition of batteries from their first life in vehicles to their second life in stationary storage or ultimate recycling.3

## Fundamental Mechanisms of Lithium-Ion Battery Degradation

The efficacy of any predictive health framework rests on a profound understanding of the electrochemical and mechanical processes that govern battery aging. Lithium-ion battery (LIB) degradation is an inherently nonlinear and complex phenomenon, influenced by operating temperature, charge/discharge rates (C-rates), depth of discharge (DoD), and charge cut-off voltages. These factors trigger irreversible physical and chemical changes within the cell, primarily categorized into the loss of lithium inventory (LLI) and the loss of active material (LAM).5

### Solid Electrolyte Interphase (SEI) Layer Dynamics

A primary driver of capacity fade and internal resistance increase is the growth of the solid electrolyte interphase (SEI) layer on the anode surface. While a stable SEI layer protects the electrode from further electrolyte decomposition, its continuous growth consumes active lithium ions and blocks ion transport.6 SEI growth is widely considered to be limited by a diffusion process. In a one-dimensional model extending from the negative electrode surface at  to , the solvent concentration  within the porous SEI is governed by the following diffusion equation 6:

Where the solvent flux  is defined by Fick's law:

At the electrode-SEI interface (), the solvent is consumed by the SEI growth reaction , often assumed to be proportional to the surface concentration 6:

The thickness of the SEI layer  grows at a rate proportional to this reaction flux, governed by the partial molar volume of the reaction products  6:

This creates a moving boundary problem typically solved using the Landau transformation.6 Under low-temperature or high-rate charging, lithium ions may deposit on the surface as metallic lithium (lithium plating) rather than intercalating, potentially forming dendrites that cause internal short circuits.

### Mechanical Stress and Structural Aging

Mechanical stressors significantly impact degradation. Phase transitions during lithium insertion/desorption in chemistries like NMC or LCO lead to strains and microcracks, reducing  diffusivity. These mechanical losses are often modeled alongside SEI growth to predict long-term capacity fade. Understanding these mechanisms allows for the extraction of health features (HFs) from incremental capacity (IC) and differential voltage (DV) curves, which serve as primary inputs for data-driven models.

## State of Health (SoH) and Remaining Useful Life (RUL) Frameworks

Managing battery assets requires accurate estimation of State of Health (SoH) and Remaining Useful Life (RUL). SoH is the ratio of current maximum capacity to initial nominal capacity 9:

EV batteries are conventionally retired when SoH reaches 80%. RUL prediction forecasts the remaining cycles before this threshold is hit. Methodologies have evolved from Equivalent Circuit Models (ECMs) to sophisticated deep learning architectures that learn nonlinear degradation directly from operational data.

## Mathematical Foundations of LSTM in Battery Prognostics

Recurrent Neural Networks (RNNs), specifically LSTMs, are uniquely suited for battery health prediction due to their gating mechanisms that mitigate the vanishing gradient problem.10

### LSTM Gating Mechanism

The core of the LSTM is the cell state , regulated by three logic gates 12:

- Forget Gate (): Decides which information from the previous state  to discard.13

- Input Gate (): Determines which new values to update.12

A candidate cell state  is also generated:

- Cell State Update: Combines the gates to produce the new internal state 14:

- Output Gate (): Regulates the information passed to the hidden state  14:

## Literature Review of Key Research Papers

The following table reviews 20 seminal research papers contributing to health prediction and EOL decision frameworks for EV batteries.

| Reference | Year | Models Used | Features Extracted | Results/Metrics |
| --- | --- | --- | --- | --- |
| Singh et al. 15 | 2025 | OptiCNN-SLSTM | QC, IR, Temperature | Oxford RMSE: 0.67%. |
| Liu et al. 16 | 2026 | FourierGNN | Spatio-Temporal features | NASA B5 RMSE reduction: 52%.16 |
| Gu et al. 17 | 2025 | DynaGPNN-SAM | Relational Topology | B0005 RMSE: 0.0104.17 |
| Nie et al. 18 | 2026 | GRU-Transformer | Temporal + Physics Priors | MAE < 1.3% (5-shot).18 |
| Xu et al. 9 | 2024 | SGEformer | IC Analysis | NASA MAE: 0.01%.9 |
| Wu et al. 15 | 2025 | AttentiveLSTM | Multi-Task SOH/RUL | Surpasses standalone models. |
| Singh et al. 15 | 2025 | Opt-KAN-XAI | V, I, T, Cycle Index | Robustness in rapid fading. |
| Zeng & Chu 19 | 2025 | Residual-aware STGNN | SCADA Signals | Superior anomaly detection. |
| Singh et al. 20 | 2026 | Transformer-KAN | HF1–HF5 | 20% RMSE improvement. |
| Sravanthi et al. 21 | 2024 | Ensemble Trees | V, I, T | Improved stability/generalization. |
| Zhao Li et al. | 2026 | LiBrain (LLM-RAG) | Telemetry + Knowledge | 92% status prediction success. |
| Wang et al. 15 | 2025 | Dual-View LSTM | Voltage signals only | MAE < 0.51%. |
| Sun et al. 15 | 2024 | LSTM-PINN | Physical Priors | NASA MAE: 0.594%. |
| Yu et al. | 2026 | Dual-Layer Ensemble | Small-sample data | High robustness in complex aging. |
| Xu et al. | 2026 | IGOOSE-OSELM | Online Sequential ML | RMSE < 0.0045. |
| Tomar et al. 15 | 2025 | AttentiveLSTM | Non-linear patterns | Surpasses current SOTA. |
| Brasil et al. 22 | 2026 | RNN, LSTM, AAS | Telemetry + History | Technical DPP blueprint. |
| Peng et al. 15 | 2025 | GNN | Partial discharge dynamics | Spatio-temporal modeling. |
| Mou et al. 15 | 2024 | CNN-BLSTM | Capacity signals | Enhanced non-linear capture. |
| Kumar et al. 15 | 2023 | SVR, BiLSTM | Logistics + Provenance | 11-44% transport savings. |

## Multi-Criteria Decision-Making for End-of-Life Strategies

Upon reaching retirement, a battery enters a decision phase where its pathway—second-life, remanufacturing, or recycling—must be determined.1

### The TOPSIS Methodology

TOPSIS ranks alternatives based on their distance to a Positive Ideal Solution () and a Negative Ideal Solution () 23:

- Normalized Matrix ():

- Weighted Matrix ():

- Distance Calculation: Euclidean distance to  () and  ().24

- Relative Closeness ():

An alternative with  is the preferred EOL strategy.25 Hybrid frameworks like AHP-TOPSIS are increasingly used to prioritize reverse supply chain alternatives, with decentralized/hybrid recycling often ranking highest in cost-sensitive contexts.4

## The Digital Product Passport (DPP) and AAS

Under EU Regulation 2023/1542, battery passports are mandatory for EV batteries (>2 kWh) by February 2027.1 The Asset Administration Shell (AAS) provides the standardized digital representation for these "digital twins".27

| Milestone | Requirement | Category |
| --- | --- | --- |
| Aug 2024 | Access to BMS SoH parameters for owners.1 | EV, LMT, Industrial. |
| Feb 2025 | Mandatory carbon footprint declaration.2 | EV Batteries. |
| Feb 2027 | Mandatory Digital Battery Passport (DPP).2 | EV & Industrial (> 2kWh). |
| Aug 2028 | Recycled content minimums (e.g., 16% Cobalt).2 | EV, SLI, Industrial. |

The DPP records static data (chemistry, manufacturing) and dynamic data (charging cycles, temperature extremes).29 Pilot implementations, such as the 2024 Volvo EX90 battery passport, demonstrate using blockchain to track degradation and environmental footprint throughout a 15-year lifecycle.

## Techno-Economic Analysis of Second-Life Applications

The economic viability of second-life battery energy storage systems (SLBESS) is driven by repurposing costs and residual value.31 SLBESS can achieve 49.2% higher NPV and 13.8% lower operational levelized cost of storage (LCOS) than new batteries.31 Projects typically break even when second-life costs are less than 60% of new battery prices.33 LFP batteries are particularly robust, retaining over 40-75% of their initial market value at retirement.32

## Conclusion

The integration of deep learning (LSTM, GNNs) with decision science (TOPSIS) within the DPP ecosystem creates a transparent framework for battery lifecycle management. This architecture ensures safety, regulatory compliance, and economic optimization, transforming retired batteries from waste into strategic assets for the circular economy.

#### Works cited

- Regulation (EU) 2023/1542 on batteries and waste batteries - UNECE, accessed April 28, 2026, https://unece.org/sites/default/files/2024-10/4_EU_Rev.1.pdf

- EU New Battery Regulation (EU) 2023/1542 | TÜV Rheinland, accessed April 28, 2026, https://www.tuv.com/landingpage/en/eu-new-battery-regulation-eu-2023-1542/

- Framework for Selecting the Most Effective State of Health Method for Second-Life Lithium-Ion Batteries: A Scientometric and Multi-Criteria Decision Matrix Approach - MDPI, accessed April 28, 2026, https://www.mdpi.com/1996-1073/18/6/1527

- Evaluation of reverse supply chain strategies for end-of-life EV batteries in Vietnam using AHP–TOPSIS - ResearchGate, accessed April 28, 2026, https://www.researchgate.net/publication/399455391_Evaluation_of_reverse_supply_chain_strategies_for_end-of-life_EV_batteries_in_Vietnam_using_AHP-TOPSIS

- Mathematical Model for SEI Growth under Open-Circuit Conditions - Scholar Commons, accessed April 28, 2026, https://scholarcommons.sc.edu/cgi/viewcontent.cgi?article=2172&context=eche_facpub

- Creating a Simple Model for SEI Growth - PyBaMM documentation, accessed April 28, 2026, https://docs.pybamm.org/en/latest/source/examples/notebooks/creating_models/6-a-simple-SEI-model.html

- A Comparison of Standard SEI Growth Models in the Context of Battery Formation - Pure, accessed April 28, 2026, https://pure-oai.bham.ac.uk/ws/portalfiles/portal/248096530/pdf.pdf

- Mathematical Model for SEI Growth Under Open-Circuit Conditions - Scholar Commons, accessed April 28, 2026, https://scholarcommons.sc.edu/etd/6828/

- Battery State of Health Estimation Using LLM Framework - arXiv, accessed April 28, 2026, https://arxiv.org/html/2501.18123v1

- LSTM - Derivation of Back propagation through time - GeeksforGeeks, accessed April 28, 2026, https://www.geeksforgeeks.org/dsa/lstm-derivation-of-back-propagation-through-time/

- (PDF) Prediction of State-of-Health and Remaining-Useful-Life of Battery Based on Hybrid Neural Network Model - ResearchGate, accessed April 28, 2026, https://www.researchgate.net/publication/383922134_Prediction_of_State-of-Health_and_Remaining-Useful-Life_of_Battery_Based_on_Hybrid_Neural_Network_Model

- Accurate Prediction Approach of SOH for Lithium-Ion Batteries ..., accessed April 28, 2026, https://www.mdpi.com/2313-0105/9/3/177

- The Math Behind LSTM - Towards Data Science, accessed April 28, 2026, https://towardsdatascience.com/the-math-behind-lstm-9069b835289d/

- SOH-KLSTM: A Hybrid Kolmogorov-Arnold Network and LSTM Model for Enhanced Lithium-Ion Battery Health Monitoring - arXiv, accessed April 28, 2026, https://arxiv.org/html/2509.10496v1

- Prediction of SOH and RUL for Li-Ion Batteries in EV Based on AttentiveLSTM Multi-Task Model - ResearchGate, accessed April 28, 2026, https://www.researchgate.net/publication/392379594_Prediction_of_SOH_and_RUL_for_Li-Ion_Batteries_in_EV_Based_on_AttentiveLSTM_Multi-Task_Model

- New Fourier graph neural network could improve lithium-ion battery ..., accessed April 28, 2026, https://www.eurekalert.org/news-releases/1124237

- State of Health Estimation for Batteries Based on a Dynamic Graph ..., accessed April 28, 2026, https://www.mdpi.com/1996-1073/18/20/5333

- Battery health prediction under data scarcity: A cross-domain ..., accessed April 28, 2026, https://www.researchgate.net/publication/399305491_Battery_health_prediction_under_data_scarcity_A_cross-domain_physics-informed_5-shot_framework_with_GRU-Transformer

- Residual-aware health prediction of power transformers via ..., accessed April 28, 2026, https://journals.plos.org/plosone/article?id=10.1371/journal.pone.0332381

- Estimation of Lithium-Ion Battery SOH Based on a Hybrid Transformer–KAN Model - MDPI, accessed April 28, 2026, https://www.mdpi.com/2079-9292/14/24/4859

- Ensemble Learning-Based Battery Health Estimation: A Comparative Study of RUL, SOC, and SOH Prediction Using Machine Learning Models - Asian Research Association, accessed April 28, 2026, https://journals.asianresassoc.org/index.php/irjmt/article/view/4709

- Circular Economy in Battery Recycling Market Growth 2026-33 - DataM Intelligence, accessed April 28, 2026, https://www.datamintelligence.com/research-report/circular-economy-in-battery-recycling-market

- TOPSIS - Wikipedia, accessed April 28, 2026, https://en.wikipedia.org/wiki/TOPSIS

- A Data Pre-Processing Model for the Topsis Method - SciSpace, accessed April 28, 2026, https://scispace.com/pdf/a-data-pre-processing-model-for-the-topsis-method-2er6trsvt6.pdf

- TOPSIS Method Overview and Steps | PDF | Mathematics - Scribd, accessed April 28, 2026, https://www.scribd.com/document/791358352/TOPSIS

- EU battery passport regulation requirements - Circularise, accessed April 28, 2026, https://www.circularise.com/blogs/eu-battery-passport-regulation-requirements

- Asset Administration Shell (AAS): What It Is and How It Organizes Product Data - explitia, accessed April 28, 2026, https://explitia.com/blog/asset-administraton-shell-aas-what-is-it/

- Introduction | Catena-X - Library, accessed April 28, 2026, https://catenax-ev.github.io/docs/Jupiter/standards/CX-0143-UseCaseCircularEconomyDigitalProductPassportStandard/introduction

- Digital Battery Passport - Use Case Guideline of the AAS - IDTA, accessed April 28, 2026, https://industrialdigitaltwin.org/en/wp-content/uploads/sites/2/2026/02/IDTA_Catena-X_Guideline_Digital_Battery_Passport.pdf

- Digital Battery Passport Guide (2024) - PicoNext, accessed April 28, 2026, https://piconext.com/en-IN/article/digital-battery-passport/

- Techno-Economic and Environmental Viability of Second-Life EV Batteries in Commercial Buildings: An Analysis Using Real-World Data - MDPI, accessed April 28, 2026, https://www.mdpi.com/2313-0105/11/11/412

- Fair Market Valuation of Electric Vehicle Batteries in Second Life Applications - MADOC - Uni Mannheim, accessed April 28, 2026, https://madoc.bib.uni-mannheim.de/67745/1/FMV%20EV%20August%202024.pdf

- Techno-economic model of a second-life energy storage system for utility-scale solar power considering li-ion calendar and cycle, accessed April 28, 2026, https://krichlab.ca/wp-content/uploads/2021/10/Mathews-et-al.-2020-Technoeconomic-model-of-second-life-batteries-for-copy.pdf